LAB REPORT

Laboratory exercise 5: Finite State Machines

3220104688 杨佳昕

12 Dec, 2024

1. Problem Description

Part I

Design a finite state machine. Whenever w=1 or w=0 for four clock rising edges, the value of z is 1, otherwise, z is 0. After four rising edges, if w remains constant, the value of z has to be 1 until w changed.

A state diagram for this FSM is shown below. Match y(8 downto 0) to ledr(8 downto 0) and z to ledg.

Use sw0 as active-low reset, sw1 as w and key0 as clock input, and design the project. And what’s more, if changing the state code table to the table below, change the vhdl code to cover the functions above.

Part II

A suggested skeleton of the FSM vhdl code has given below. Fill the code to achieve the functions same as task 1 in Part I.

library ieee;

use ieee.std_logic_1164.all;

entity part2 is

port(…define input and output ports…);

end part2;

architecture behavior of part2 is

…declare signals…

type state_type is (A, B, C, D, E, F, G, H, I);

signal y_Q, y_D : state_type; -- y_Q is present state, y_D is next state

begin

…

process(w, y_Q)

begin

case y_Q is

when A if (w = ‘0’) then y_D <= B; else y_D <= F; end if;

… other states…

end case;

end process;

process(clock)

begin

…

end process;

end behavior;

2. Design Formulation

Part I

The key point to both tasks are the logical relationship of A to I.

In task 1, every time the state is updated forward, y loops one bit forward. We can use ssl expressions.

In task 2, the last bit was kept still, and the other bits of y loop. Keep the last bit unchanged, throw the first bit to the second-to-last bit, and move other bits forward. The process is shown in the graph below.

Once the relationship is achieved, match each state to the next state with w value with case sentences.

Part II

The first process is the same as the match part above. The second process is to refresh y_Q value with y_D value when clock or reset triggers. After the two parts, match the state with led values to determine whether led lights or not.

NB: Reset is set as active low, so when reset = 0, the whole system changes to the original state.

3. Design Entry

Part I

When the state code table changes, we just add a function shift to keep the last bit still and loop the other bits:

Other codes are similar to those above.

Part II

4. Simulation and Synthesis Results

Part I

Ledg triggers:

Unable to trigger:

Reset:

When state code table changes:

Ledg triggers:

Unable to trigger:

Reset:

Part II

Ledg triggers:

Unable to trigger:

Reset:

5. Experimental Results

Part I

Part II

6. Discussion and Conclusion

Part I asks to write a logical expression that drives the flip-flop input ports. However, in the code given above:

This is not logical expression such as Boolean expressions. The correct code of this part is:

The uniqueness of this code is the processing logical of variables in process expressions. The value of any variable will not change until the next process triggered. Thus, although we declared state(0) <= ‘0’ and state(1) <= state(0) and not w, before the second process triggered, the value state(0) = 1 is successfully transferred to state(1) expression. And when the second process arrives, state(0) is refreshed to 0.

However, I had to notice that both codes above achieved the same function, e.g., state transfer. It seems that the first expression (regarded wrong because of no logic) is more easy to read and more ‘logical’ in human eyes.